Application Note 27

Data Analysis of Tube Formation Assays

1. Introduction.........................................................................................................  2

2. Choosing the right system..................................................................................  3

3. Data Analysis Overview ...................................................................................... 3

4. Pre!experimental work........................................................................................  4

4.1. Optimization of Parameters..........................................................................  4

4.2. Establishing positive and negative controls .................................................. 6

4.3. Experimental Plan and Number of Repeats..................................................  6

4.4. Documentation.............................................................................................  6

5. Data Analysis ......................................................................................................  6

5.1. Evaluation of pictures ................................................................................... 6

5.2. Data processing............................................................................................  7

6. Data Interpretation ..............................................................................................  8

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 1 of 9

Application Note 27

1. Introduction

The tube formation assay is a powerful tool for the

screening  of  substances  in  order  to  discover  their

anti!  or  pro!angiogenic  effect  on  cultured  cells.  In

comparison  to control cell cultures, the effect of a

substance can be measured in parameters, like such

as the length of the tubes build developing on the

gel surface or number of loops.

Performing     tube     formation     assays     requires

optimization of the practical protocol, as well as the

establishment  of  a  data   acquisition  that  provides

comparable  data. The three critical key parameters

are  the   time   point  of   data  acquisition,   the  cell

seeding density, and the serum concentration in the

cell culture medium.

This  application  note  will  help  determine  the  best

parameters  for  tube  formation  assays  using   your

own   cell  line,   thus   ensuring   the  acquisition   of

reliable and reproducible results.

In some parts of this document, we discuss µ!Slide

Angiogenesis,  but  all  types  of  tube   formation

assays  will  show  the  same  characteristics  and

can be handled in the same way. For the specific

handling of µ!Slide Angiogenesis, please refer to

Application Note 19

NOTE: The expression „tubes“ describes the cords of cells that are visible in

a  formed  network.  It  does  not  mean,  specifically,  that  the  cords  have  a

lumen.

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 2 of 9

Application Note 27

2. Choosing the Right System

The  first step, when starting with tube formation experiments, is to find the right

experimental system. It has to be relevant, in context to your approach, and the data

must be experimentally accessible. The three most important considerations are:

•    Cell Type—tube formation is physiologically observed in endothelial cells

•    Gel Matrix—it needs to be compatible with the cells and it must provide

binding motifs for cell attachment

•    Medium Composition (growth factors requested?)

A setup that should work with most endothelial cells consists of Matrigel   , Growth

TM

Factor!Reduced, and any endothelial cell medium with 2% serum or less.

3. Data Analysis Overview

For  a  better   understanding  of  the  next   steps  of  the  protocol   optimization,  an

overview is needed of the parameters included in the data analysis.

A microscopic picture, such as the one below, shows a tube formation network with

four detectable key parameters:

•    Cell covered area (blue)

•    Tubes (red)

•    Loops (yellow)

•    Branching points (white)

By  using  those  values,  a  range  of

further   values   can   be   calculated,

such  as  the  mean  tube  length,  the

total  tube  length,  the  mean  area  of

loops, and others.

There  are  several  possible  ways  to  analyze  your  pictures.  One  way  is  to  do  this

manually with an image manipulation software, such as ImageJ.

Another option is to use the WimTube image analysis tool, which calculates the key

parameters  of  the  network  by  an  automated  algorithm.  We  recommend  using  this

web!based tool for maximum reproducibility and objectivity. In addition, it gives you

results within minutes. For a free trial of WimTube image analysis, please visit the

.

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 3 of 9

Application Note 27

4. Pre/Experimental Work

Before  starting   with  your   screening  experiments,   it  is  absolutely   necessary  to

optimize the experimental procedure and setup.

4.1. Optimization of Parameters

Some parameters, such as cell seeding density, time points, or serum concentration,

strongly affect the data values. Therefore, the creation of a strict protocol is crucial

for comparability of data.

4.1.1. Time Intervals for Measurement

First, choose a medium cell concentration and a tube formation!promoting setup to

record a time curve. For HUVEC ~10,000 cells per well, using Matrigel with reduced

growth factors and a medium without serum or growth factors is adequate.

Next, prepare the gel and cell suspension as described in Application Note 19, “Tube

Immediately after seeding the cells on

the gel surface, place the slide into an incubation chamber on the microscope and

start a time!lapse recording. Take a picture every 10 minutes by adjusting the focus

at each time point with a software!based tool. Alternatively, if live cell imaging isn’t

possible, then take a picture every hour, for at least the first 8 ! 10 hours, and then a

final picture after an overnight incubation period.

Finally,  analyze  the  pictures  with  WimTube  and  visualize  the  curve  with  suitable

software. The time curve of a parameter (e.g., total tube length) typically looks like

the  chart  shown  below.  In  the  first  hours  the  curve  rises  to  a  maximum,  then

declines to a plateau phase (beginning at ~7 hours), and finally then slowly flattens

out (>20 hours). The  characteristics of all four key parameters look similar. For this

reason, the evaluation of only one parameter is sufficient. We chose the tube length

as a parameter, because it is controlled easily by comparing the original pictures to

the evaluated ones.

The best results are found at the maximum phase and also in a stable phase that

does not decline too fast. In the example above, the 5!hour mark and 10!hour mark

would be good reference points for your measurements.

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 4 of 9

Application Note 27

4.1.2. Cell Seeding Density

For determining the optimal cell seeding density, record the characteristics of your

cell line.

First, make a dilution series in the range of 5!40 x 10        cells/ml and then seed the

3

cells onto the gel surface. Incubate the slide for the determined length of time, after

following the instructions in section 4.1.1., and then take phase contrast pictures.

Make at least five repeats per cell concentration.

Then, evaluate the images, as mentioned in Sections 3 and 5, respectively. Visualize

the data in a graph, as shown below. The data will once again show a characteristic

curve  with  a  maximum  value.  This  maximum  marks  the  best  seeding  density  for

your cell type.

4.1.3. Serum Concentration

The  addition of serum to the culture medium     might  influence the tube formation

behavior. In most cases, the serum inhibits tube formation. To avoid this problem,

try different serum concentrations, from 0% up to 20%, with the parameters you

defined in Sections 4.1.1 and 4.1.2. This will help you decide which concentration is

best for tube formation and cell survival.

4.1.4. Magnification

The magnification determines the dimension of the representative screen section that

will be shown on the chip of the camera. Because tube formation is a process that

uses  the  whole  surface  of  the  well,  it  is  important  to  image  the  widest  possible

section. If you are not interested in detecting cellular details, it is     recommended to

use a small magnification of 4x or 5x .  When this is not possible, you can also stitch

images that have a higher magnification.

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 5 of 9

Application Note 27

4.2. Establishing Positive and Negative Controls

To create a positive control, choose a setup where tube formation is assured (e.g.,

Matrigel  with  reduced  growth  factors  and  serum!free  medium  for  HUVEC).  The

positive control will ensure that the cells are healthy and also that any anti!angiogenic

effects that are observed are being caused by the investigated substances.

To  create  a good  negative  control,  choose a  substance  that  is  proven  to have  an

inhibiting  effect  on  tube  formation  development  (e.g.,  suramine  for  HUVEC).  The

negative control will ensures that the cells are not inert to inhibition and will give a

comparable value to the values of your experiment.

4.3. Experimental Plan and Number of Repeats

Before starting the whole experimental series, calculate the amount of the required

material, such as cells, media, gel matrix, and substances, as well as the laboratory

requirements and the time schedule.

For  a  stable  statistical  analysis,  a  minimum  of  four  independent  experiments  is

recommended,  each  with  a  minimum  of  8  single  wells.  The  required  number  of

repeats is dependent on the homogeneity of the data. Following a strict protocol is

crucial for data acquisition. The p!value of the Students!T!test will provide feedback,

if your data is homogeneous enough.

4.4. Documentation

A reasonable documentation includes all of the parameters that where determined

in this section. Generate a tabular chart in which every experiment is recorded in

one column. An example is given in the appendix.

5. Data Analysis

5.1. Evaluation of Pictures

The   microscopic   pictures   need    to   be   analyzed,   in   order   to    quantify   the

characteristics of the network. This can be done by hand with an image processing

software, or by sending the pictures to the automated image analysis at Wimasis

GmbH.

A free trial of 10 images is available, so that you can evaluate the image analysis

performance. Please follow these steps to test your own images:

1.

Create an account at:

https://mywim.wimasis.com/index.php?page=Launch&select=Tube_Formation&gr=ibidi

Activate your account by confirming of the registration email.

2.

3.

4.

Login at: https://mywim.wimasis.com/index.php?page=Home&gr=ibidi

Upload  your  images   after  choosing  the   right  magnification  range  (see

Section 4.1.4). The magnification level affects the expected values for the

cell dimensions, in terms of the given thresholds.

5.

The following graphical formats are accepted:

*.jpg, *.jp2, *.png, *.gif, *.tiff, *.tif, *.bmp

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 6 of 9

Application Note 27

6.

After a few minutes, the results will be ready to be downloaded. You will

receive the following:

•    single images with colored cell area, tubes, nodes, and loops

•    a  summary.csv!file  with  all  of  the  measured  and  calculated  values

The key metrics are

!

!

!

!

!

Cell covered area [%]

Total tube length [px]

Total number of branching points

Total number of loops

Total number of nets (connected network of tubes)

All of the key metrics will show the same characteristic over the experimental span of

time, therefore it is sufficient to evaluate only one parameter. The most stable and

traceable value is the total tube length. Therefore, we recommend evaluating only the

tube length as a representative value for the formation of tubes (see also 4.1.1).

5.2. Data Processing

Each analyzed well generates one tube length value. The values of one experiment

(minimum  8  wells  per  condition)  are  then  summed  up  and  a  standard  deviation  is

calculated.  The  standard  deviation  should  not  be  more  than  10%.  This  is  just  one

experimental data point.

The  left  graph  shows  the  distribution  of  single  well  results  for  one  experimental

condition, at four different time points.

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 7 of 9

Application Note 27

For a stable statistical analysis, at least four data points for each condition are needed.

Please keep in mind that each data point consists of 8 individual wells.

The single data points are grouped into a mean value and can be displayed in a bar

plot.  Statistical   analysis  is   done  by   a  Student’s   T!test,  in   which  the   different

populations of data points are tested against each other.

6. Data Interpretation

The  Student’s  T!test  gives  evidence  of  whether  or  not  the  two  separate  samples,

with t!distributed values, differ significantly from each other. This will depend on the

number of repeats. It is recommended to perform the T!test with suitable statistical

software or in Microsoft Excel.

It is important to consider what can realistically be predicted with the tube formation

data.  Tube  formation  is  a  very  complex  process  that  combines  a  huge  variety  of

biochemical  reactions   and  pathways.  In  our   opinion,  this  experimental   setup  is

suitable for a screening of substances, and for a preliminary prediction of whether or

not the substance influences tube formation processes. It does not explain how this

influence  works.  For  further  investigation,  more  profound  biochemical  analysis  will

need to be done.

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 8 of 9

Application Note 27

Appendix:

Documentation Example

Date

Conducted by:

08.12.2011

HW

Experimental Parameters:

Cell type:

HUVEC

Passage:

P2

Medium:

EC Medium without supplements

Concentration Cell Suspension

Confluence before Seeding:

Gel matrix:

2x105 c/ml

70%

Matrigel with GFs

5 min

Time in Suspension (max. 15 min):

A1

A2

A3

A4

A5

B1

B2

B3

B4

B5

C1

C2

C3

C4

C5

negative control

negative control

negative control

negative control

negative control

positive control

positive control

positive control

positive control

positive control

Microscope/Imaging

Microscope:

Camera:

Image dimension in pixel:

Image dimension in mm²:

Objective:

Nikon Ti

DS!Qi1Mc

1280x2048

6.97

4x

Large Image:

1x2

Dimension of pixel (µm/px):

Number of images:

Time points:

1,61 µm/px

3

1h, 6h, 24h

Analysis

Analysis date:

Analyzed by:

Modul:

Date

Conducted by:

09.06.2011

Wimasis

wimtube low magnification

08.12.2011

HW

Application Note 27

© ibidi GmbH, Version 1.0, Helga Wagner, April 10th, 2012

Page 9 of 9